02/11/02       15:38         ©708  649  3434

Material S&.<£ty Data Sheet

SANFORD CORP

MSDSNo: 1394

Page 1 of 2

i!002/003

Sanford Corporation

2740 Washington Boulevard

Bellwood	IL	60104

Telephone No: Initiated By: Date of Last Revision: Medical Emergency No:

1-800-323-0749 Susan Nyborg June 1,2001 1-800.228-5635

Section 1 - Product identification

Product Name:	Liquid Paper Pen and Ink11 Correction Fluid

Colors:	White

Includes: Liquid

All components of this product comply with the U.S. Toxic Substances Control Act (TSCA).

Section 2 - Composition

Titanium dioxide (13463-67-7), solvent naphtha (64742-89-8), mineral spirits (64742-48-9), resins, dispersants, colorants, mustard oil (57-06-7), masking fragrance

Section 3 • Physical / Chemical Characteristics

Boiling Point:

Vapor Pressure (mm Hg): Specific Gravity: Solubility In Water: Appearance / Odor: Evaporation Rate:

For product unless otherwise Indicated

209-230 °F

124mmHgat10D°F

1.29

Negligible

White fluid; solvent odor

3.6 (butyl acetate = 1)

Section 4 - Fire and Explosion Hazard Data

Flash Point (Method Used): Flammability Limits (% by volume):

Extinguishing Medium:

Special Fire Fighting Procedures:

25.5°F (Closed Cup) Lower:     1%

Upper:    8%

Dry chemical, foam, carbon dioxide.

In fires involving large quantities, use self-contained breathing apparatus. Cool fire-expose containers with water spray/fog.

Unusual Fire and Explosion Hazards:        Product is flammable. May produce hazardous decomposition products.

1/02  15:39

©70S 649 3434

SANFORD CORP

\

111003/003

MSDS No.

1394 Page 2 of 2

Section 5 - Reactivity Data

Stability:

Conditions to Avoid: Chemical Incompatibility: Hazardous Decomposition: Hazardous Polymerization:

Stable

Avoid contact with open flame or other ignition sources.

Strong oxidizers

May produce oxides of carbon, nitrogen, and various hydrocarbons in fire.

Will not occur.

Section 6 - Health Hazard Data

Chemical Listed as Carcinogen or Potential Carcinogen:

IARC Monographs:	No

National Toxicology Program:	No

OSHA Regulated:	No.

Routes of Entry: Inhalation - None under normal use conditions. Eyes - Irritation may occur. Wash through with water. Skin - Wash with soap and water. Ingestion - Contact a physician. Note to physician: Contains solvent naphtha and mineral spirits which may cause chemical pneurnonftis.

NOTES: If vapors are deliberately inhaled, the following symptoms may occur: respiratory irritation, dizziness,
drowsiness, headache, nausea, unconsciousness, convulsions, cardiac sensftization, coma, and death. Mustard oil
added to deter abuse,	ORM-D Consumer Commidity

Section 7 - Precautions for Safe Handling and Use

Steps to be Taken in Case Material is Released or Spilled:

Waste Disposal Method:

Precautions to be Taken in Handling and Storing:

Other Precautions:

Wipe up with absorbent materials.

Discard In accordance with Federal, State, and Local Regulations. In large quantities, store in a well-ventilated area. Product is flammable.

Section 8 - Personal Protection and Exposure Control Measures

Eye Protection; Skin Protection: rtespiratory Protection:

Ventilation: 'rotective Clothing:

| HMIS Code |  |
| --- | --- |
| Health | 2 |
| •lammability | 3 |
| Reactivity | 0 |
| 'ersonal Protection | N/A |
| 3 - Minimal /4 = Severe Hazard |  |

None under normal use conditions. None under normal use conditions. None under normal use conditions. None under normal use conditions. None under normal use conditions.

Sanford Corporation has been advised by council that the OSHA Hazard Communciation Standard does not apply to the Sanfard Product described in this MSDS. The reason for the exemption is contained in 29 CFR Part 1910.1200 (b)(6)(«), as amended July 1, 1994 per the Code of Federal Regulations. The information contained in this MSDS ts forwarded to you for your information, but is not meant to imply that the product is covered by the Hazard Communication Standard nor is the MSDS meant to comply with all requirements of the Hazard Communication Standard.