rCLOROX

Clorox Professional Products Company

1221 Broadway Oakland, CA 94612 Tel. (510)271-7000

Material Safety Data Sheet

| .Product: commercial solutions™ ultra clorox® germicidal bleach i 1 |  |
| --- | --- |
| Description: clear, light yellow liquid with chlorine odor |  |
| Other Designations Distributor Emergency Telephone Nos. |  |
| Clorox Sales Company F°r Medical Emergencies Call: EPA Reg. No. 67619-8 1??1 r d (800)446-1014 Sodium Hypochlorite Solution Broadway For Transportation Emergencies Chemtrec Oakland CA94612 (800)424-9300 |  |
| II Health Hazard Data | III Hazardous Ingredients |
| DANGER: CORROSIVE. May cause severe irritation or damage to eyes and skin. Harmful if Swallowed. The following medical conditions may be aggravated by exposure to high concentrations of vapor or mist; heart conditions or chronic respiratory problems such as asthma, chronic bronchitis or obstructive lung disease. Some clinical reports suggest a low potential for sensitization upon exaggerated exposure to sodium hypochlorite, particularly on damaged or irritated skin. Routine clinical tests conducted on intact skin with Clorox Liquid Bleach found no sensitization in the test subjects. Under normal consumer use conditions the likelihood of any adverse health effects are low. FIRST AID: | Ingredients Concentration Worker Exposure Limit |
|  | Sodium hypochlorite 6.0-7.35% not established CAS #768 1-52-9 Sodium hydroxide <0.2% 2.0 mg/m3 TLV-C a CAS #131 0-73-2 aTLV-C = ACGIH Threshold Limit Value-Ceiling Limit None of the ingredients in this product are on the IARC, NTP or OSHA carcinogen list. |
| EYE CONTACT: Rinse with plenty of water for at least 1 5 minutes. Get |  |
| prompt medical attention. SKIN CONTACT: Wash skin thorouahlv with soap and water. |  |
| r-% |  |
| .ysician or poison control center immediately. INHALATION: If breathina problems develop remove to fresh air. | V Transportation and Regulatory Data |
|  | U.S. DOT Hazard Class: Not restricted |
| IV Special Protection and Precautions | U.S. DOT Proper Shipping Name: Hvpochlorite solution with not more than |
|  | 7% available chlorine. Not Restricted per49CFR172.101(c)(12)(iv). |
| Hvaienic Practices: Wear safety classes. With repeated or prolonged use. | EPA - SARA Title III/CERCLA: |
| wear nitrile, neoprene, or butyl rubber gloves. Wash after contact with product. Avoid breathing vapors. Engineerina Controls: Use aeneral ventilation to minimize exposure to vapor |  |
|  | Bottled product is not reportable under Sections 31 1/312; contains no chemicals regulated under Section 313; and contains chemicals (sodium hypochlorite < 7.35% and sodium hydroxide < 0.2%) which are regulated under Section 304/CERCLA. TSCA Status' All component of this product are on the TSCA Inventory. |
| or mist. Work Practices: Avoid eve and skin contact and inhalation of vapor or mist. |  |
| KEEP OUT OF THE REACH OF CHILDREN. |  |
| VI Spill Procedures/Waste Disposal | VII Reactivity Data |
| Spill Procedures: Absorb and containerize. Wash residual down to | Stable under normal use and storage conditions. Strong oxidizing agent. Reacts with other household chemicals such as toilet bowl cleaners, rust removers, vinegar, acids or ammonia containing products to produce hazardous gases, such as chlorine and other chlorinated species. Prolonged contact with metal may cause pitting or discoloration. |
| sanitary sewer. Contact the sanitary treatment facility in advance to assure ability to process washed-down material. Waste disposal: Dispose of in accordance with all applicable federal, state |  |
| and local regulations. |  |
| VIII Fire and Explosion Data | IX Physical Data |
| Not flammable or explosive. In a fire, cool containers to prevent rupture and release of sodium chlorate. * | Boiling point 212°F100°C decomposes) |
|  | Specific Gravity (H2O=1 ) 1 .085 |
|  | Solubility in Water complete pH 11.4 |

®1983, 1991 THE CLOROX COMPANY     " :.

DATA SUPPLIED IS FOR USE ONLY IN CONNECTION WITH OCCUPATIONAL SAFETY AND HEALTH

DATE PREPARED  10/00