MATERIAL SAFETY DATA SHEET

POLYWOOD STICK

-

(O465O7-O126P -4O183576-221OO1OO)|DATE OF ISSUE	SUPERSEDES

|	02/23/90_	_/._/

SECTION I - GENERAL INFORMATION

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A	POLYWOOD STICK

CHEMICAL FAMILY	FORMULA

EPOXY STICK	X<--MIXTURE

MANUFACTURERS  NAME:

DYNA SYSTEMS A PARTSMASTER CO DIV OF NCH

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

P.O. BOX 655326

DALLAS, TEXAS 75265-5326

PREPARED BY:	! PRODUCT CODE NUMBER ! EMERGENCY TELEPHONE NUMBER

RICHARD STOLLEY/T.S.CHEM.      221OO1OO	214-438-1381

SECTION II- HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS.

CHEMICAL NAME (INGREDIENTS)  :

BASE: DIGLYCIDIAL ETHER OF BISPHENOL A

HAZARD	>IRR/SENS      TLV	>5 MG/M3 2.    PEL-->N/A

STEL(TWA)* ->NOT EST     | CAS#-->25O85-99-8 |	;

CHEMICAL NAME (INGREDIENTS)  :

2,4,6 TRKDIMETHYLAMINOMETHYL	PHENOL) *

HAZARD	>IRRITANT      TLV	>5PPM  3.      PEL-->NOT EST.

STEL(TWA)* ->NOT EST	j CAS#-->9O-72-2

SECTION III - PHYSICAL DATA

(CONTINUED)

SECTION III - PHYSICAL DATA

PAGE : O2

BOILING PT.(FAHRENHEIT)  |N/A

SPEC GRAVITY (H20=1 )

:O.9

VAPOR PRESSURE (MM HG).  | N/A

in/a

VAPOR DENSITY (AIR=1)

in/a

PH

PERCENT, VOLATILE BY VOLUME (%)

|0

COLOR

AMBER

MERCAPTAN LIKE

OPAQUE

in/a

EVAPORATION RATE (BU AC = 1 )

SOLUBILITY IN WATER VISCOSITY

NEG. PUTTY

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT (METHOD USED) A:>25O FT.C.C.

FLAMMABLE LIMITS

in/a

in/a

UEL

EXTINGUISHING MEDIA  "ALCOHOL"
X<--FOAM	X<--FOAM     X<--C02

DRY X<--CHEMICAL

WATER <--SPRAY

<--OTHER

SPECIAL FIRE FIGHTING PROCEDURES

WEAR NIOSH/MSHA APPROVED RESPIRATOR WHEN FIGHTING FIRES IN CONFINED AREAS

TO PROTECT FRAOM ACRID SMOKE, FUMES AND HAZARDOUS DECOMPOSITION PRODUCTS.

UNUSUAL FIRE & EXPLOSION HAZARDS NONE KNOWN

NFPA HAZARD RATING (O=INSIGNIFICANT; 1=SLIGHT;2 = MODERATE;3 = HIGH;4 = EXTREME) : 2 <--HEALTH    1 <--FLAMMABILITY   O <--REACTIVITY      <--SPECIAL

SECTION V - HEALTH HAZARD DATA

^

THRESHOLD LIMIT VALUE :

NOT ESTABLISHED FOR MIXUTRE SEE SECTION II

EFFECTS OF OVEREXPOSURE

-	ACUTE -    (SHORT TERM EXPOSURE)

PROLONGED OR REPEATED EXPOSURE MAY	CAUSE SKIN SENS IT IZATION. IN SENSITIVE

INDIVIDUALS CONTACT MAY CAUSE SKIN	IRRITATION.   THIS PRODUCT IS BOTH

POLYWOOD STICK

(CONTINUED)

SECTION V - HEALTH HAZARD DATA

MECHANICALLY IRRITATING AND CHEMICALLY IRRITATING  TO THE EYES. INHALATION IS NOT A HAZARD IN NORMAL USE HOWEVER THIS PRODUCT GIVES OFF TOXIC FUMES WHEN BURNED OR WELDED. INGESTION MAY LEAD TO GASTRIC IRRITATION WITH NAUSEA AND VOMITING.

- CHRONIC -  (LONG TERM EXPOSURE) PROLONGED SKIN CONTACT CAN CAUSE SENSITIZATION.

PRIMARY ROUTE OF ENTRY:   <-- INHALATION  <-- INGESTION  X<-- ABSORPTION

EMERGENCY & FIRST AID PROCEDURES

INHALATION :

GENERALLY NOT A HAZARD. MOVE TO FRESH AIR AND TREAT SYMPTOMATICALLY. GIVE OXYGEN IF BREATHING IS DIFFICULT AND ARTIFICIAL RESPIRATION IF BREATHING HAS STOPPED.  GET MEDICAL ATTENTION.

EYE CONTACT:

FLUSH WITH  WATER FOR 1O MINUTES. LIFTING UPPER AND LOWER LIDS OCCASONALLY.   GET MEDICAL IRRITATION IF IRRITATION PERSISTS. TREATMENT IS MOST EFFECTIVE WITHIN ONE MINUTE.

SKIN CONTACT:

WASH WITH SOAP AND WATER. SENSITIVE INDIVIDUALS SHOULD APPLY AN EMOLLIENT RICH CREAM AND SHOULD SEE A PHYSICIAN_IF_IRRITATION PERSISTS.

INGESTION :

DO NOT INDUCE VOMITING. GIVE ONE OR TWO GLASSES OF WATER AND CALL PHYSICIAN ASPIRATION OF MATERIAL INTO THE LUNGS DUE TO VOMITING CAN CAUSE CHEMICAL PNEUMONITIS WHICH CAN BE FATAL.

NOTES TO PHYSICIAN : GASTRIC LAVAGE IN INDICATED.  DO NOT_INDUCE VOMITING.

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN BY:

IARC   <--YES  NTP    <--YES  OSHA   <--YES  ACGIH   <--YES  OTHER   <--YES
X<--NO         X<--NO	X<--NO	X<--NO	X<--NO

2,4,6 TRI(DIMETHYLAMINO METHYL PHENOL)

ORL-RAT LD5O: 2.5 G/KG 1.        SKN-RAT LD5O: 128O MG/ KG 1. SKN-RBT LD5O:  1 .2G/KG  1.        SKN-RBT 5OOMG/24H SEVERE  1.

EYE-RET 5OOMG/24H/SEVERE  1. DIGLYCDIAL ETHER OF BISPHENOL A SKN-RBT SOO MG OPEN MILD 1. EYE-RBT 2 MG/24 H SEVERE 1. ORL-RAT LD5O: 11 G/KG 1.

SECTION VII - REACTIVITY DATA

POLYWOGC STICK

(CONTINUED)	SECTION VII - REACTIVITY DATA	PAGE : O4

|X<--STABLE     <--UNSTABL |  CONDITIONS TO AVOID
STABILITY

OPEN FLAMES, LONG-TERM EXPOSURE TO HIGH TEMPERATURES.

INCOMPATIBILITY (MATERIALS TO AVOID)

STRONG OXIDIZING AGENTS OR STRONG BASES OR MINERAL ACIDS

HAZARDOUS DECOMPOSITION PRODUCTS

OXIDES OF CARBON, NITROGEN, SULFUR, ACIDS, ALDEHYDES.

WILL NOT         MAY  ! CONDITIONS TO AVOID HAZARDOUS      X <--OCCUR      <--OCCUR   '

POLYMERIZATION N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

STEPS TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

DUE TO THE NATURE OF THE PACKAGING OF THIS PRODUCT A SPILL IS UNLIKELY.

WASTE DISPOSAL METHOD

DISPOSE OF IN ACCORDANCE WITH FEDERAL, STATE AND LOCAL POLLUTION

REGULATIONS.

NEUTRALIZING AGENT N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

REQUIRED VENTILATION

LOCAL EXHAUST RECOMMENDED TO CONTROL VAPOR LEVELS IF BURNNED OR WELDED. GENERAL EXHAUST IS MORE THAN ADEQUATE TO PROTECT WORKER FROM EXPOSURE TO LEVELS ABOVE PERMISSIBLE EXPOSURE LIMITS.

RESPIRATORY PROTECTION

WEAR NIOSH APPROVED RESPIRATOR TO PROVIDE ADEQUATE VENTILATION

WHEN BURNING, WELDING OR MACHINING(GRINDING) .

PROTECTIVE GLOVES

RUBBER GLOVES SHOULD BE WORN BY SENSITIVE

INDIVIDUAL DEPENDING ON SEVERITY OF EXPOSURE.

EYE PROTECTION

SAFETY GOGGLES SHOULD BE WORN DEPENDING ON

SEVERITY OF EXPOSURE.

POLYWOOD STICK

(CONTINUED)	SECTION IX - SPEC'.AL PROTECTION INFORMATIOPAGE : OS

OTHER PROTECTION

CLEAN, PROTECTIVE CLOTHING SHOULD BE WORN.

SECTION X - STORAGE AND HANDLING INFORMATION

| STORAGE 1OO F < | TEMPERATURE --MAX 3O F | < | --MIN | ! INDOOR I * | 1 HEATED I | [refrigerated! ¦x ] | OUTDOOR |
| --- | --- | --- | --- | --- | --- | --- | --- |

PRECAUTIONS TO BE TAKEN IN HANDLING & STORING AVOID CONTACT WITH EYES, SKIN AND CLOTHING.

OTHER PRECAUTIONS

WASH THOROUGHLY AFTER HANDLING. KEEP OUT OF REACH

OF CHILDREN. AVOID CONTACT WITH EYES, SKIN AND

CLOTHING.

SECTION XI - REGULATORY INFORMATION

CHEMICAL NAME	C.A.S. NUMBER         U??ER. °/°_LIMIT

N/A

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REQUIREMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHOR I ZATION ACT OF 1986 AND 4O CFR PART 372.

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT, END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FOR ROUTINE JANITORIAL WORK, OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES), OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

CALIFORNIA PROPOSITION 65

WARNING: THIS PRODUCT CONTAINS THE FOLLOWING CHEMICAL(S) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE (1) CANCER OR (2) BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM NONE

SECTION XII - TRANSPORTATION * (FOR FUTURE USE)

APPLICABLE REGULATIONS

<--49 CFR    <--IMCO      <--TARIFF 6 D    <--IATA   <--MILITARY AIR (AFR 71-

SHIPPING NAME

POLYWOOD STICK

(CONTINUED)       SECTION	XII - TRANSPORTATION * (FOR FUTURE USE)      PAGE : O6

HAZARD CLASS	|  ID NUMBER | REPORT QTY

LABELS	| LIMITED QTY

UNIT CONTAINER

DOT  SPS CONTAINER	| NET EXPLOSIVE WT.

AEROSOL PROPELLANT(S)

SECTION XIII - REFERENCES

- DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS, 6TH EDITION,
N.  IRVING SAX.

- THRESHOLD LIMIT VALUE FOR CHEMICAL SUBSTANCES IN THE
WORK ENVIRONMENT, 2ND EDITION, ACGIH,  1985.

- VENDOR'S MSDS.

*SHORT TERM EXPOSURE LIMIT (TWA) LISTED AS FINAL RULE LIMITS PUBLISHED IN THE FEDERAL REGISTER/VOL.54 NO.12,  1-19-89

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION, HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

DYNA SYSTEMS A PARTSMASTER CO DIV OF NCH ASSUMES NO RESPONSIBILITY FOR PERSONAL INJURY OR PROPERTY DAMAGE CAUSED BY THE USE , STORAGE OR DISPOSAL OF THE PRODUCT IN A MANNER NOT RECOMMENDED ON THE PRODUCT LABEL. USERS ASSUME ALL RISKS ASSOCIATED WITH SUCH UNRECOMMENDED USE, STORAGE, OR DISPOSAL OF THE PRODUCT.